# Autonomous Navigation Tutorial

Using Nav2 and Clearpath Robotic Platforms

Applied Research Center Onboarding (2025)

Authors

Sebastian Vargas

Tyler Coy

# Table of Contents

# Prerequisites for Tutorial

For this tutorial, users will learn how to install, set up, and operate a robot using autonomous navigation. Why might you be asking? Because Anthony told you to, so don’t be a savage and learn. In all seriousness, autonomous navigation is a cornerstone of modern robotics, especially within nuclear environments. As researchers at the Applied Research Center (ARC), we want to apply the latest practical technologies onto our platforms. Additionally, we want to minimize the work done during platform deployments. By having our robots move by themselves it frees up the operator to better focus on data acquisition and other tasks being performed by the robot. That being said, although this is a tutorial, there are some basics you must know prior to continuing further.

As the wise Anthony Abrahao said, “If you were born, if you breath, if you know how to drink water, you know python.” This tutorial expects users to have a basic to intermediate understanding of Python as well as C++. Additionally, the navigation packages use the Robotic Operating System (ROS2), so users must also know the basics of ROS2 development, terminology, and execution. Udemy has a great course on ROS2 basics that will really help if you still do not fully understand it. A pdf book is also available that helps better understand transforms (THESE ARE SUPER ESSENTIAL). Ask one of your “elders” (someone that’s been here for more than a year) for a copy of it.

If you feel comfortable with all the prerequisites (Python, C++, & ROS2) proceed at your own risk.

# Safety

You will be handling robotic platforms that have the potential to inflict a lot of damage. These robotic platforms have been given nicknames to affectionately show their capabilities:

Husky A200: The Shin Buster.

Jackal J100: The Ankle Breaker.

These platforms produce a lot of torque and can move fast, so improper handling will lead to injury. Thankfully, careful handling is based on common sense. Things to keep in mind are:

## Raise the platform on stilts during development

Sometimes changes in code can cause unwanted movements even when there is no input to the robot. It is always a good idea to prevent movement by raising robots off the ground.

## Be Ready to Hit the E-Stop

Most robotic platforms have emergency stop buttons. In figure 1 below you can see the husky E-stop while figure 2 shows the motor cutoff for the jackal. The algorithm, may freak out and make your robot move erratically. If that happens, you must be ready to approach it carefully and hit the E-Stops.

Figure1. Husky E-Stop

Figure2. Jackal Motor Cutoff

## Always Check Your Max Speed

There are parameters inside each robot and navigation algorithms that limit how fast the platform can move. Be sure to double check the number and the units to ensure the robot is moving at a safe speed.

## Keep Test Area Clear

Don’t break anyone’s shins while operating the robot. That is really mean. When testing movement, ensure that the robot has a clear area to operate in and notify everyone around that you will be moving the robot. If someone enters unexpectedly, be ready to stop the robot.

## Disconnect Battery When Done

Batteries may fail. Always disconnect them from the robotic platform when done. That preserves the battery's health and prevents catastrophic failures (catching fire, exploding, etc.) when no one is around.

## Always Check What You Are Plugging In

Always make sure you know the correct voltage before you start plugging stuff into a voltage source. Someone who will not be named almost plugged 24 volts into a component when it could only handle 9 volts. This would have fried the component and would have delayed the project. So, double and triple check your voltages before plugging in and powering components.

## When in Doubt, Power Off

If you ever don’t like how a robot is moving or just feel uneasy about an aspect or component, it's best just to power off the robot. This is way better than crashing into something or damaging a component.

# Why Use Navigation?

If you skipped safety, go back and read it you savage. Well, if you can't navigate how the heck do you know where you are at? If you don't know your surroundings and don't know the next steps and waypoints, you are blind. Navigation is the key to any mobile robotic platform. You have to scan your surrounds for obstacles and the next path forward and be wary of any obstacles in the way.

# Navigation Stack Workflow?

Navigation starts by turning on the LiDAR and verifying that it publishes data. Step one: open RViz and make sure you can actually see the scans. If RViz is blind, your robot is blind.  RVIZ is an open-source, 3D visualization tool for the Robot Operating System (ROS) that allows developers to visualize sensor data, robot models, and system status in real-time

The navigation stack has three major components:

## 1. LiDAR Driver

You must use the power of the lidar, whether it be 2D or 3D to get a scan of the room around you. This is the software that powers the LiDAR. Without the driver, the LiDAR is a paperweight. The driver handles serial ports, connection, scan modes, and publishes the /scan topic used by SLAM and navigation.

## 2. SLAM Toolbox

This is where the SLAM map is created. SLAM Toolbox takes LiDAR scans and odometry and builds a 2D map of your environment. You can save this map and reuse it later for localization and autonomous navigation. The lidar creates a SLAM. SLAM stands for simultaneous localization and mapping. In English, this means that you are locating yourself and making a map at the same time. A slam map is a gray diagram of the area/room you scanned. You should be able to recognize what you are scanning based on the layout around you. Make sure there are no overlaps or mistakes. If there are overlaps or boundaries that make no sense, it is best to scan again. The robot, whether the Husky or Jackal, will have the lidar attached, and the robot will follow the data the lidar collects. You will use RVIZ to visualize what the lidar sees and the map being scanned. You can set  waypoints based on this slam map or program for the robot to move in a certain pattern based on the slam map.

## 3. Nav2 Bringup

This launches the core Navigation Stack. It can include:

- The map you built

- AMCL for localization

- The robot state publisher and transforms

- Costmaps

- Global and local planners

- Controllers

- And even simulation tools like Gazebo if needed

All three parts must work together for navigation to run smoothly.

## Global Costmap (The Big Picture Map)

The global costmap is built from your saved SLAM map. It represents the entire environment—all walls, rooms, hallways, obstacles, and everything the robot mapped earlier.

The global costmap is used by the global planner, which creates paths such as:

- “Go from the lab entrance to the back corner.”

- “Navigate through this hallway and into the next room.”

It does not care about moment-to-moment obstacles or small details. It cares about long-distance route planning, just like Google Maps. It looks at the map, draws a path, and gives the robot a clean roadmap from Point A to Point B.

If the global costmap is wrong, the robot will try to drive through a wall. Not ideal.

## Local Costmap (The Robot’s Immediate Vision)

The local costmap focuses on what’s happening right in front of the robot.
 It updates in real time from LiDAR scans.

The local costmap is used by the local planner, which handles:

- Avoiding people walking by

- Avoiding chairs, tables, or objects not in the SLAM map

- Steering around obstacles

- Refining motion

- Making micro-adjustments

This behaves like the robot’s “short-range eyesight.”
 Even if the global planner says “go straight,” the local costmap may say:

“Wait, there's a chair in front of me, let me go around it.”

This prevents the robot from smashing into things you forgot to include on the map.

If the local costmap is bad, the robot will jitter, freeze, or ram into things. Again, not ideal.

# How to Flash the Robot?

You must locate the robot's computer under the hood. The computer on the robot should have USB ports. If you are not sure which computer you have on your robot, ask an experienced amigo and they can tell you. To flash the robot, you need to use a USB stick. Remember, if you flash anything on the USB stick, all of the data on the USB will be lost FOREVER and cannot be recovered. So be sure there is nothing on the USB stick that you will need in the future. You need BalenaEtcher software to be able to put the software on the USB stick.  just google BalenaEtcher and the website to download it will come up. The software the jackal uses is ros2 Humble. You have to first download the image that you want to download. Make sure that you look up which ros2 image the robot is compatible with, or you will waste hours of your time and many headaches wondering why the software won't boot up on the robot. Also, only flash the command line interface (CLI), the version that only uses the terminal. Do not put a graphical user interface (GUI) on a robot. This is not good practice. You should be interfacing with your robot through your computer via ssh and not using the robot’s onboard computer directly. That is how it's done in the real world because often times you are not able to always plug something into the robot. So back to BalenaEtcher, once the image is downloaded, select the target, which is the USB stick and click flash. DOUBLE CHECK THAT YOU HAVE THE USB STICK SELECTED. If you accidentally target your computer’s hard drive, say goodbye to everything and get ready to be publicly shamed.  Then wait for the software to be loaded onto the USB stick. After the flashing is complete, plug the USB stick into the port on the robot’s computer. Press “delete” as the computer boots up, select the boot option that has the USB stick and then, badabing badaboom, the software should begin loading onto the robot. You will need to stay and answer the prompts the installer asks you. Once the software installs, the robot will turn off. Remove the USB and then turn on the robot, log in (the stock password should be ‘clearpath’) and run the installer you find in the home directory.

Insert installer command for terminal here

The installer will ask more questions, so be present to answer them.

# Setting Up Robot

The robot will come with factory settings. We will require some minor modifications to continue the project. Namely, we need to change the ROS_DOMAIN_ID and IP address so the robot can communicate with your host machine (desktop).

# Create Workspace?

The workspace is a Nav2 workspace.

Okay, so you made your Nav2 workspace folder. Great. But right now, that folder is about as useful as a Jackal with no battery. Until you build the workspace, ROS 2 has no idea your files even exist. It’s like trying to talk to someone who has AirPods in — you can yell all you want, but nothing gets through.

This is where colcon comes in. Colcon is basically the robot’s compiler-slash-brain activator. When you run a colcon build, it looks at everything inside your workspace and says, “Ah yes, I see the chaos you’ve created, let me turn this into something functional.” After the build, your workspace gets an install folder. That folder is where all the real, usable navigation files end up — configs, executables, launch files, the whole package.

Here’s the thing: if you don’t colcon build, nothing will work.
 You can write the prettiest YAML files in the world and it won’t matter. The Jackal will remain a dumb brick until you build the workspace and formally introduce it to ROS.

After the build finishes successfully (assuming you didn’t break anything… again), you now have to source the workspace. Sourcing is basically tapping the robot on the shoulder and saying, “Hey, use this version of things from now on.” If you don’t source the workspace, ROS will act like your navigation package doesn’t exist and pretend you never made anything.

The moral of the story?
 Make the workspace → build the workspace → source the workspace.
 If you forget any of these steps, good luck, you’re going to be staring at error messages wondering why the robot refuses to move.

# Sourcing the Workspace (.bashrc)

Alright, you built the workspace. Nice. But here’s the part everyone messes up at least once: sourcing the workspace.
 If colcon build is the brain surgery, then sourcing is waking the patient up afterward. If you don’t source, the workspace just sits there like a vegetable, doing absolutely nothing while you scream at the terminal.

Sourcing tells ROS 2, “Use this workspace. These are my configs. These are my launch files. This is the version of reality I want you to believe.” If you skip this step, ROS 2 will happily ignore all your hard work and run the default system setup instead. And then you’ll wonder why the robot is driving sideways or why Nav2 keeps exploding.

Normally, you source the workspace manually by typing the source command in the terminal. But you’re not going to want to do that eighty times a day. You’re going to forget, you’re going to run the wrong launch file, and you’re going to break something.

That’s why we use .bashrc.

Your .bashrc file is basically your terminal’s morning routine. Every time you open a new terminal window, .bashrc runs automatically. So if you put your source commands in there, your workspace will always be activated without you having to think about it. It’s like telling your terminal, “Brush your teeth and activate Nav2 every morning.”

Once you add your workspace sourcing to .bashrc, every terminal you open will already know about your navigation setup. No more guessing, no more forgetting, no more wondering why half your packages won’t run. It just works. (Unless you broke something. In that case… good luck.)

Point is:
 Build the workspace → Source the workspace → Put the source in .bashrc → Stop suffering.

## Common issues/bugs

Make sure that you set the ROS Domain ID the same for both your robot and host computer. If you don't know the ID, open your bashrc file. If one is not set, you must set export ros domain id on the robot and your desktop to the same number.

If you type ros2 (tab) and nothing starts to show you probably need to source the package you are working on.

## RVIZ / Nav Workflow

In rviz for waypoint navigation make sure that you have the lidar running, you should see the points filling the window. Then start your slam scan. Drive around the room and complete a slam scan. Make sure it looks accurate and nothing overlaps. In rviz, turn, select lidad, select map, and select robot model. Once the slam map is finnished save it using map_server map_saver_cli and give the file a name. If you dont do this you will lose the slam map and will have to scan all over again. Once the slam map is saved, open it and select points using the 2d pose estimate. The robot should follow according to the layout of the slam map. Chick on the map where you want the robot to and which way it will face. On the map as the robot is moving you will see a grey floor with black circles on it. The black circles represent an object in the way. You cannot put a waypoint on a black circle. The gray floor means open space on the map that can be navigated.

# Launching the LiDAR (Jackal Commands)

Before any kind of mapping, visualization, or navigation can occur, the robot must have an accurate and continuous stream of laser scan data. Without the LiDAR publishing, the Jackal has absolutely no sense of its surroundings — it is essentially a blind shopping cart with a battery.

To wake up the LiDAR, run the following command on the Jackal itself:

### Launch RPLIDAR A2M8 Driver

ros2 launch rplidar_ros rplidar_a2m8_launch.py

This command initializes the RPLIDAR node, connects to the correct serial port, and begins publishing data to the /scan topic. If this node fails to launch, RViz will have nothing to visualize, SLAM Toolbox will refuse to build a map, and Nav2 will act confused (and by confused, we mean absolutely useless).

If you receive errors about ports, permissions, or missing device files, it means your LiDAR either did not mount properly or is not connected at all. Always confirm that /dev/ttyUSB0 or the equivalent device appears on boot.

# Transform Remapping & Static Transforms

The ROS2 navigation ecosystem relies heavily on TF — a transform tree that explains how every sensor, link, and coordinate frame relates to every other frame. If your TF frames are incorrect, misaligned, missing, or inconsistent, your robot will behave as if the LiDAR is floating somewhere in deep space.

### Launch the Transform Remapper (Jackal)

ros2 launch nav transform_remap.launch.py

This launch file remaps TF data into the expected frame structure used by Nav2 and SLAM Toolbox. Many Clearpath robots (including the Jackal) require a bit of massaging to get their transforms aligned with what the navigation stack expects.

Once transforms are correctly remapped, you can safely begin your SLAM or navigation session.

### Start SLAM / Nav Bringup

ros2 launch nav online_async_launch.py

This launches SLAM Toolbox in online_async mode, which is the recommended mode for scanning new environments. The robot will begin constructing a SLAM map in real-time, fusing laser scans with odometry.

# Aliases (Convenience Commands)

Typing long ROS2 commands repeatedly is a guaranteed way to introduce typos and destroy your sanity. That’s why aliases exist — they condense a massive command sequence into a single short phrase.

You have a helpful alias on the Jackal:

nav_robotics_lab

This alias automatically sources the correct environments and launches your navigation stack. Instead of typing five lines of commands, you simply run this alias and let the robot do the heavy lifting.

# Desktop-Side Commands (RViz & TF Helpers)

Some commands should only be run on the desktop, not the robot. These commands help visualize transforms, support debugging, and provide critical data needed during mapping and navigation.

### Run TF Remapper on the Desktop

ros2 run nav tf_remapper

This adjusts TF data locally on the desktop, ensuring RViz displays accurate frame relationships. If frames appear disconnected or floating, this command often resolves it.

### Static Transform Publisher

If a sensor frame is missing or incorrectly positioned, you can manually publish a static transform:

ros2 run tf2_ros static_transform_publisher 0 0 0 0 0 0 base_link laser

This command is the “manual override” for frame correction. It forces ROS to treat laser as being directly above base_link with no rotation or offset.

If your robot doesn’t know where its LiDAR is located, everything else falls apart.

# RViz Setup (Desktop Only)

RViz is the main visualization tool used during development. If RViz cannot see the robot model, LiDAR data, or transforms, then Nav2 cannot run properly.

Launch RViz:

rviz2

### After opening RViz:

- Add RobotModel

- Add LaserScan

- Add Map

- Add TF

Then change the Fixed Frame to:

odom

This ensures that all data is aligned with the robot’s odometry frame.

Under the LaserScan display, manually type:

laser

RViz autocomplete is unreliable — typing it yourself ensures it maps to the correct scan topic.

To visualize SLAM activity:

ros2 launch nav online_async_launch.py

# Robot Network Configuration (robot.yaml)

Correct network settings are essential for communication between your desktop and the Jackal. All ROS2 nodes depend on matching domain IDs and proper networking configurations.

Open the robot configuration file:

sudo nano /etc/clearpath/robot.yaml

Inside this file, update:

- The robot’s IP address

- The ROS_DOMAIN_ID (must match your desktop)

- Jackal controller settings

Restart Clearpath services:

sudo systemctl restart clearpath-platform.service

This reloads the Jackal’s internal topic publishers. If you don’t restart, none of your changes will take effect.

# Nav2 Parameters – mapper_params_online_async.yaml

This YAML file contains the parameters used by SLAM Toolbox during online mapping. Incorrect values here will cause SLAM to behave unpredictably.

Verify:

- Frame names

- Scan topic

- Map resolution

- SLAM mode

If something looks strange in your map, the problem is almost always inside this YAML.

# Useful Debugging Commands

These are essential tools for diagnosing problems.

### View TF frames:

ros2 run tf2_tools view_frames

This generates a PDF showing your entire TF tree.

### List parameters of nodes:

ros2 param list /rplidar_node
ros2 param list /slam_toolbox

### Dump parameters:

ros2 param dump /slam_toolbox

### Check joystick teleop:

ros2 topic echo /joy_teleop/cmd_vel

If this topic stays empty, your controller isn’t connected or recognized.

# TMUX Workflow (Highly Recommended)

TMUX allows you to run multiple terminal sessions without opening dozens of windows. It is extremely useful when launching multiple ROS2 nodes simultaneously.

### Split windows:

Horizontal:

ctrl+b "

Vertical:

ctrl+b %

Move between panes:

ctrl+b + arrow keys

Resize:

ctrl+b + hold arrow keys

This setup makes running LiDAR, SLAM, Nav2, TF, and RViz processes much more manageable.

# Fresh Session Workflow

Whenever you log into the robot and want to start fresh:

### Source your workspaces:

source ~/ros2_workspaces/rplidar_ws/install/setup.bash
source ~/ros2_workspaces/nav_ws/install/setup.bash

### Publish static TF:

ros2 run tf2_ros static_transform_publisher 0.125 0.0 0.2 0 0 0 base_link laser

### Launch LiDAR:

ros2 launch rplidar_ros rplidar_a2m8_launch.py

### Open RViz:

rviz2

Set the fixed frame to odom.

If topics fail to appear:

sudo systemctl restart clearpath-platform.service

# Saving the Map

After completing your SLAM scan, save the map:

ros2 run nav2_map_server map_saver_cli -f <map_name>

If you forget to save, the map disappears the moment SLAM stops running. You will have to scan the entire room again.

# Launching Navigation With Saved Map

Launch Nav2 using your saved map:

ros2 launch nav2_bringup bringup_launch.py use_sim_time:=false map:=/home/administrator/robotics_lab_map1.yaml params_file:=/home/administrator/ros2_workspaces/nav_ws/src/nav/config/nav2_params.yaml

### Make sure:

- Every use_sim_time value in the params file is set to false

If this is wrong, Nav2 will refuse to run correctly.

# Do NOT Use VS Code Over SSH While Running SLAM

VS Code consumes large amounts of network bandwidth. Running it during SLAM causes:

- TF delays

- LiDAR desync

- RViz lag

- Maps becoming corrupted

Use plain terminal during active SLAM operations.

# RViz – Add Costmap

In RViz, you should see:

- Global Costmap

- Local Costmap

- Map

- TF

- Laser Scan

Expect frame names:

map
odom
base_link
laser

If you see dozens of extra frames, your TF configuration is wrong.

# .bashrc Setup Notes

Add these lines so your workspaces always build automatically:

source /etc/clearpath/setup.bash
source /home/administrator/ros2_workspaces/rplidar_ws/install/setup.bash
source /home/administrator/ros2_workspaces/nav_ws/install/setup.bash

Add alias:

alias nav_launch_robotics_lab='ros2 launch nav online_async_launch.py'

Now every terminal is ready for robotics work instantly.

# Transform Remapper Launch File Notes

Open:

ros2_workspaces/nav_ws/src/nav/launch/transform_remap.launch.py

Inside:

- Z offset = 0.2

- X offset = 0.125

- Yaw = value of pi

- Comment out the remapper section (Jackal no longer uses namespaces

# SSH Access to Jackal

ssh administrator@192.168.1.221

Password: clearpath

If you cannot connect, check your subnet and ROS domain ID.

## Verifying Topics and Data Flow

Once you’ve launched everything, don’t just assume it’s working because there are no errors. You should always verify that topics are alive and that data is flowing through the system like it should. ROS2 is like a moody teenager: quiet doesn’t necessarily mean “fine.”

### List All Active Topics

On the Jackal (or the desktop if you’re remoting into the robot):

ros2 topic list

You should see things like:

- /scan – LiDAR data

- /tf and /tf_static – transform frames

- /cmd_vel – velocity commands

- /joy_teleop/cmd_vel – joystick teleop output (if controller is active)

- /slam_toolbox/... – SLAM topics

- /map, /global_costmap/..., /local_costmap/... – Nav2 topics

If /scan is missing → LiDAR isn’t running.
 If /map is missing → SLAM/Map server isn’t up.
 If /cmd_vel is silent while navigation is active → something is broken in the Nav2 pipeline.

### Check Joystick Input

To verify that your Logitech controller or other joystick is actually being read:

ros2 topic echo /joy_teleop/cmd_vel

Move the joystick sticks/triggers and watch for velocity messages.
 If nothing prints, the joystick is either:

- Not recognized

- Not mapped correctly

- Plugged into the wrong machine

No /joy_teleop/cmd_vel = no manual override.

## Deep Dive: Nav2 Params, use_sim_time, and Costmaps

nav2_params.yaml is where a lot of silent failures hide. If Nav2 seems to hang, spam warnings, or refuse to start, you probably messed something up in here (or forgot to change the sim time).

There are two main places these params can live:

- System install (stock Nav2):

sudo nano /opt/ros/humble/share/nav2_bringup/params/nav2_params.yaml

- Your Nav2 workspace (recommended override):

nano ~/ros2_workspaces/nav_ws/src/nav/config/nav2_params.yaml

### use_sim_time Must Be False

For real robots (Jackal in ARC lab), you are not using Gazebo sim time. You must make sure:

- In the system file (if you’re using it directly):

sudo nano /opt/ros/humble/share/nav2_bringup/params/nav2_params.yaml

- In your workspace override:

nano ~/ros2_workspaces/nav_ws/src/nav/config/nav2_params.yaml

Change every instance of:

use_sim_time: true

to:

use_sim_time: false

Miss one, and you’ll spend an hour wondering why everything is paused in ghost time.

### Costmap Recording and Behavior

Once use_sim_time is fixed and Nav2 is running properly, the global and local costmaps will:

- Fill in obstacles from /scan

- Respect inflation radius

- Block planning through occupied cells

If the costmap never fills in, or obstacles don’t show up:

- Check that the observation_sources in the costmap sections reference the right topic (scan).

- Check that frame names match: map, odom, base_link, laser.

Bad costmaps = robot either acts scared and doesn’t move, or goes full Leroy Jenkins into obstacles.

## From SLAM Scan to Full Navigation: Workflow Summary

Here’s a high-level navigation workflow you can follow so you don’t forget steps and then blame the robot.

First lauch transfrmation

Then do

### 1. Start LiDAR and SLAM (on Jackal)

If an error , restart the jackal -

sudo systemctl restart clearpath-platform.service

ros2 run tf2_ros static_transform_publisher 0.125 0.0 0.2 0 0 0 base_link laser

ros2 launch rplidar_ros rplidar_a2m8_launch.py
ros2 launch nav online_async_launch.py

Drive the Jackal around the lab and let SLAM Toolbox build a complete map. Don’t half-scan the room; incomplete maps become pain later.

### 2. Visualize Everything in RViz (on Desktop)

If you need to use docker because of the new system, use these commands

sudo docker run -it --rm \

--name nav2_desktop \

--net=host \

-e ROS_DOMAIN_ID=67 \

-e DISPLAY=$DISPLAY \

-v /tmp/.X11-unix:/tmp/.X11-unix:rw \

-v /home/$USER/nav_ws:/home/ros/nav_ws \

-v /home/$USER/rplidar_ws:/home/ros/rplidar_ws \

osrf/ros:humble-desktop-jammy \

Bash

If this above gives issues, copy this one line code, change the name of the container if necessary

sudo docker run -it --rm --name nav2_desktop --net=host -e ROS_DOMAIN_ID=67 -e DISPLAY=$DISPLAY -v /tmp/.X11-unix:/tmp/.X11-unix:rw -v /home/$USER/nav_ws:/home/ros/nav_ws -v /home/$USER/rplidar_ws:/home/ros/rplidar_ws osrf/ros:humble-desktop-jammy bash

Make sure there is no space after the word bash or there will be a syntax error

Then,

source /opt/ros/humble/setup.bash

rviz2

Rviz2

Do this command ros launch nav2 online_async_launch.py

In the jackal do ros2 launch nav_bringup_launch.py
enter shortcut nav_robotics_lab_ something just to nav_r tab

Nav_launch_robotics_lan

In RViz:

- Set Fixed Frame → map

- Add RobotModel

- Add LaserScan (topic: scan / frame: laser)

- Add Map

- Add TF, odom, base_link, front_0_mount

- (Optionally) add costmaps later once Nav2 is running

Make sure the map looks like the real room—no overlapping walls, weird double corridors, or ghost obstacles.

In rviz2

To tf- for frames, click base_link, click front_0_mount, click odom

Click add topic, in the by topic section, click the Map in /costmap

### 3. Save the Map

When you’re happy with the SLAM result:

ros2 run nav2_map_server map_saver_cli -f robotics_lab_map1

This creates robotics_lab_map1.pgm and robotics_lab_map1.yaml.
 If you forget this step → start all over tomorrow.

### 4. Launch Nav2 with the Saved Map

Now you stop SLAM and switch to localization + navigation:

### 5. Send Goals in RViz

In RViz (with Nav2 bringup running):

- Use 2D Pose Estimate to localize the robot on the map (this may not be necessary, if you click 2d goal post and it moves the right way, then just go from there)

- Use 2D Goal Pose to send navigation goals

Watch the global and local costmaps populate, and the robot should follow clean paths that respect obstacles.

## Editing nav2_bringup Launch Logic (Advanced)

If you really need to change Nav2’s default behavior or swap out params/bringup structure, you can edit the launch file directly (this is more advanced — do it carefully).

System launch file:

sudo nano /opt/ros/humble/share/nav2_bringup/launch/bringup_launch.py

Things you might change here:

- Default params_file path

- Default use_sim_time value

- Whether certain nodes launch by default

Warning: changing system install files can get messy when you upgrade ROS or share instructions with others. Whenever possible, override behavior via your own workspace launch files instead of editing system ones.

## Extra Param Debugging: Static Transform and Other Nodes

If you create a static transform node with a custom name (for example via a launch file), you can also inspect or dump its parameters.

Example:

ros2 param dump /static_transform_publisher

(If your node name is different, use ros2 node list to find it first.)

This is useful for verifying:

- The exact translation (x, y, z)

- The rotation (roll, pitch, yaw or quaternion)

- That the node is publishing between the correct parent/child frames

If your TF looks wrong in RViz, this is the place to double-check what you told ROS to believe.

## Remote Visualization, Remotectl, and Performance Tips

In ARC, you’re often running heavy workloads on the robot but visualizing on your desktop. This is the correct setup, but you can still accidentally crush the network if you’re not careful.

### General Rules

- Do heavy compute (SLAM, Nav2, LiDAR, TF) on the Jackal.

- Do visualization (RViz) on the desktop.

- Use SSH or remotectl to access the robot’s terminal, not its GUI.

### Do Not Overload the Network

As mentioned:

- Do not run VS Code over SSH while SLAM + Nav2 + RViz are active.

- Heavy tools like VS Code’s remote extension can saturate bandwidth, cause latency, and make your TF/map updates laggy.

Symptom of overload:
 RViz freezes, robot delays responding, frames “jump,” and map quality degrades.

Use basic terminals for runs, and keep the fancy stuff for when the robot is idle.

## Change Tracking and “Don’t Break Future You”

Every time you edit one of these files:

- /etc/clearpath/robot.yaml

- /opt/ros/humble/share/nav2_bringup/launch/bringup_launch.py

- /opt/ros/humble/share/nav2_bringup/params/nav2_params.yaml

- ~/ros2_workspaces/nav_ws/src/nav/config/nav2_params.yaml

- ~/ros2_workspaces/nav_ws/src/nav/launch/transform_remap.launch.py

- ~/.bashrc

Get into the habit of:

- Adding a short comment with your name + date + what changed.

- Keeping a changelog text file in your nav_ws/src/nav/ folder summarizing big edits.

- Telling the next poor soul (future ARC fellow) where to look so they don’t re-debug the same thing from scratch.

Future you will appreciate that you behaved like a civilized engineer and not like a code goblin.

## Why Your Robot Refuses to Move – Debugging Checklist

At some point, you’re going to stare at RViz, see the robot, see the map, click a 2D Goal Pose… and the robot will just sit there like a stubborn brick. When that happens, don’t immediately blame Nav2 or “ROS being stupid.” There is usually a very specific reason it refuses to move, and it’s often one of a few common mistakes.

First, check whether the robot is actually receiving velocity commands. Run:

ros2 topic echo /cmd_vel

If this topic is completely silent even when you send a goal, then Nav2 is not outputting commands. That usually means something is wrong in the planner or controller configuration, or the robot is not properly localized on the map. If /cmd_vel is publishing values, but the robot doesn’t move, then the problem is on the hardware side—motors disabled, e-stop engaged, wrong controller topic, or platform services not running.

Next, make sure the global and local costmaps are not blocking movement. If the robot’s footprint is constantly inside a red or lethal area in the costmap, Nav2 will refuse to move because it thinks it’s already colliding with something. This can happen if your footprint radius is too large, your inflation radius is outrageous, or the robot is accidentally spawned partially inside a wall in the map frame. Adjust the costmap parameters and make sure the robot’s starting pose is in a sane location.

Finally, confirm that transforms (tf) are valid and up to date. If base_link, odom, and map are not being published correctly, Nav2 cannot compute paths. Use ros2 run tf2_tools view_frames and look at the generated PDF to see if your TF tree is fully connected. If you see missing links, question marks, or transform timestamps that are lagging, fix those before blaming the navigation stack. The robot isn’t ignoring you—it’s just refusing to drive blind.

## SLAM Quality Tips – How to Get a Clean Map

A navigation system is only as good as the map you give it. If your SLAM map looks like a glitchy Minecraft world like your robot was drunk, Nav2 will plan nonsense paths and get confused constantly. A clean map is not magic—there are very practical habits that make it better.

First, drive slowly and deliberately during SLAM scans. Fast, jerky, or zig-zag motion creates motion distortion in the LiDAR data and makes it harder for SLAM Toolbox to match scans. Move at a comfortable walking speed in smooth arcs and straight lines. Avoid sudden spins in place unless you need to fill in blind spots. Think of it like painting the room with the LiDAR: slow, even strokes, not wild scribbles.

Second, avoid tight loops and double-backing in small spaces too often. While SLAM can handle loop closure, constantly revisiting the same corner from weird angles increases the chance of mismatches and causes overlapping walls in the final map. Try to cover the area in a simple pattern—like a lawnmower pattern or U-shaped passes—before coming back through transitions and doorways. When you do revisit old regions, try to enter them in a similar way you scanned them originally.

Third, minimize moving obstacles during scanning. People walking around, chairs being dragged, doors opening and closing—these all appear as conflicting geometry in the LiDAR data. If possible, politely kick everyone out of the test area for the first “golden” SLAM scan, then let chaos resume later. You want the saved map to represent the stable structure (walls, pillars, permanent objects), not the random positions of humans and rolling chairs on a Monday morning.

If your map looks warped, doubled, or full of weird shadows, don’t force it. It’s faster in the long run to rescan the environment carefully than to fight a bad map through param tweaks and prayer.

## Husky vs Jackal – Practical Differences for Nav2

Even though Husky and Jackal both use Clearpath’s platform stack, they are not just “big and small versions of the same robot.” From the perspective of Nav2 and SLAM, they behave a bit differently and that shows up in how you tune and operate them.

The Husky A200 is heavier, larger, and more stable at speed. Its inertia makes it smoother when moving over rough ground but also slower to stop. For Nav2, this often means you want more conservative acceleration and deceleration limits, and slightly larger inflation and obstacle radii in the costmaps so that global and local planners don’t try to “thread the needle” through tight spaces. Husky can physically do a lot, but it’s not great at squeezing through narrow, cluttered indoor environments.

The Jackal J100, on the other hand, turns like a little murder Roomba. It can rotate in tighter spaces, start and stop quickly, and handle indoor environments more easily, but also tends to look jittery in the local costmap if your controller gains are too aggressive. For Jackal, you can usually use smaller footprint and inflation radii and slightly more agile controller parameters. However, this also means small mapping or TF errors show up more dramatically because the robot reacts so quickly.

When writing tutorials or launch files, always make sure you know which robot you are targeting. Don’t blindly copy Husky settings onto Jackal or vice versa. The underlying logic is the same, but their physical behavior, safety margins, and comfort zones are not interchangeable.

## Field Deployment Workflow – From Lab Hero to Real Site

Getting navigation working in the lab is one thing; getting it to behave at a real DOE-LM field site with dust, sun glare, weird slopes, and random junk everywhere is another level. Having a deployment checklist saves you from embarrassing moments where the robot decides to yeet itself toward a barrier in front of an audience.

Before leaving ARC, make sure you have: a working SLAM + Nav2 setup tested in the robotics lab, recent backups of your workspaces, configuration files saved to version control or at least a USB, and physical spares (cables, power strips, network adapters, etc.). Don’t assume the field site will have good Wi-Fi, extra cables, or even enough outlets. Bring your own ecosystem.

At the site, start with manual teleop and just look at /scan and /tf to confirm sensors and transforms survived the trip. Uneven ground, metal structures, and long outdoor corridors can affect LiDAR returns differently than the lab. Do a small, local SLAM test scan and verify the map makes sense before trying full autonomous navigation. If the environment is large, consider scanning it in chunks and saving multiple maps you can stitch or reuse later.

Only after you’ve verified: LiDAR data looks good, transforms are correct, SLAM maps are clean, and Nav2 respects obstacles in RViz—then start running full waypoint navigation. Don’t jump straight into fully autonomous missions with no testing. Your future self, your field team, and everyone’s shins will thank you.

## Quick Glossary: Nav2 / ROS2 Terms in Human Language

Sometimes ROS2 documentation sounds like it was written by a committee of sleep-deprived wizards. Here’s a quick translation of some terms you’ve already seen in this tutorial into normal human language.

ROS_DOMAIN_ID – Think of this like a walkie-talkie channel number. Robots and computers on the same ROS_DOMAIN_ID can hear each other’s messages. If the IDs don’t match, they might as well be in different universes.

TF / Transforms – This is the system that tells ROS where everything physically is: where the LiDAR is relative to the base, where the base is relative to odom, where odom is relative to the map, and so on. If TF is wrong, the robot is “hologram teleporting” around instead of actually knowing where it is.

SLAM – Simultaneous Localization and Mapping. Fancy phrase for: “build a map while figuring out where you are in that map at the same time.” It’s like trying to draw a floor plan of a house while walking through it blindfolded, only touching walls and furniture.

Costmap – A 2D grid where each cell represents how bad it would be to go there. Free space is cheap, obstacles are expensive, and lethal obstacles are “nope.” Nav2 uses this to decide where the robot should and shouldn’t drive.

AMCL – Adaptive Monte Carlo Localization. This is the algorithm that lets the robot figure out where it is on a pre-existing map by comparing its laser scans to that map. SLAM builds the map; AMCL helps the robot reuse it later.

Global Planner – The “big-picture” path planner. It looks at the full map and finds a route from the robot’s current position to the goal, like Google Maps drawing a route from your house to the airport.

Local Planner / Controller – The short-term brain. It uses the global plan as a guideline but focuses on the next few meters, avoiding pop-up obstacles like people, carts, or chairs while still trying to follow the overall route.

use_sim_time – A setting that tells ROS whether to use simulated time (from Gazebo or a bag file) or real wall-clock time. If this is set wrong, everything acts like time is frozen or weird, and Nav2 quietly loses its mind.

New System Setup:

If you have the new system you need to use docker because you do not have sudo access.

Have someone email you the workspace and use the unzip command. The command is unzip <path to folder>. After its unzipped make sure it was downloaded properly. Make sure the src folders are there for the workspace. Then start a docker container.

docker run -it --rm \

--name nav2_desktop \

--net=host \

-e ROS_DOMAIN_ID=67 \

-v /home/<your-username>/nav_ws:/home/ros/nav_ws \

-v /home/<your-username>/rplidar_ws:/home/ros/rplidar_ws \

ros:humble-desktop \

bash

Put the path to the containers in the –v section and make sure the ros roman id matches the jackal and your desktop.